New Super Cub Build Data Sheet

Engine: Lycoming IO-360  will be similar to a Ly-Con engine with features such as ported heads, high compression forged pistons, and cold light weight sump.  In addition, this engine’s internal rotating assembly was selected for its ability to sustain 2900 RPM continuously, with rated power projected to be 220 HP.  I planned to limit it to 2750 continuous for a conservative 200+ HP.  Includes all appropriate paperwork and yellow tags on all engine components.  Reference engine component details:

Crankshaft PN 74709, with 8130, 006 mains, 006 rods, crankshaft was superseded by 74708 and is currently listed as a  Lyc PN:13B17166 non counterweight, with thick flange.

Engine case, Wide deck Superior PN : SL3 6800 - W1 REV T, Divco conversion 2, standard cam and followers, TT 400 HR, tagged W fresh Gold Alodine dip.

Lycoming connecting rods LW 19332, (SAME AS IO 540 RODS), W interlocking caps and 2.421 Dia, overhauled at Rick Romans with 8130.  Includes new Lycoming bolts and nuts.

Lycoming Wide Deck, parallel valve cylinders PN 75874 or LW 12427 standard steel bore, 1200 HRS TTSN, Overhauled with new intake and exhaust valves, all new guides, keepers, and caps. Combustion Engineering 10 to 1 compression forged pistons and rings. Also includes new Lycoming wrist pins and buttons.

Camshaft is an overhauled Lycoming PN 76097 with PN 872877 cam followers and pushrods; all eight rocker arms were re-bushed.

Bendix fuel injection system overhauled and custom calibrated by Airflow Performance Inc.

Does not include ignition, starter, alternator, or exhaust system.

Propeller:

75” MT brand, three blade, hydraulic, constant speed propeller, and governor.  This prop has a reversing feature which makes it  very popular with seaplane users

Model # MTV-9-B-C-R(M)/CR190  No logs, 400 hours reported, excellent condition.  Due for an overhaul based upon age.  US Dealer, McFarlane Aviation Products, 682 E 1700 RD, Building C, Baldwin City, KS 66006

Airframe:

All new, 4130 chrome-moly steel, VR3 laser cut, tig welded, wide-body super cub.

This airframe was built by a professional shop in Canada.  It has all the ‘Alaska mods,’ comparable to Airframes Alaska’s ‘Willow Mountain Ranch’ wide body or Javron’s wide body super cub.

In addition, this airframe has overhead flaps, float attach fittings, doors on both sides, extended baggage, and custom turtle deck.

The airframe has been media blasted and painted with two coats of epoxy primer and two coats of ultra-black.  It is ready for cover.

Wings:

Back Country/Smith brand, factory built, all aluminum, extended (16’) wings with extended (80”) flaps and (9.5’) ailerons.  Also includes standard wing tanks and Javron sight gauges.   Includes Javron struts and all associated hardware.  Rated for 2300 lbs.

Empennage:

Over-sized tail feathers for high HP applications.  30% larger than standard.  Javron brace wires and empennage hardware.

Firewall Forward:

All new, Carbon Cub cowl, cooling baffles, oil cooler, titanium firewall, boot cowl, and hardware.  Also Includes Javron’s zero offset engine mount with swing-out fittings.

Landing Gear:

3 x 9 mains (extended gear) with 8.5 Matco wheels, tires, and brakes.  The tires, wheels and brakes have 80 HRS.  New Bearhawk, pneumatic tailwheel and fork.

Hardware

Javron brand, over-head flap handle assembly, pulleys, cables, controls, and other associated parts.

Front and rear brakes with toe brakes and master cylinders.

Windows:

Great Lakes Aero brand windshield, forward skylight, D windows, and two 4’x4’ sheets of the same for both doors.  All UV grey with Great Lakes Aero’s super cool feature.

Covering Materials:

Fresh Hipec brand covering materials (solvent based).  Includes everything needed to cover this aircraft.

Interior

Javron front and rear seat belts with ratcheting shoulder harness for PIC.

TMI Pro-Series Reclining bucket seats, front, and rear.

All materials needed for a complete carbon fiber interior, including the baggage/sleeping area.

Negotiated delivery is available within the Continental United States